RC car suspension research:

(Oscar Hook Evans)

Looking at our project, the remote controlled trophy-truck, we can see that it utilises an independent suspension system, in conjunction with adjustable shocks to allow for the suspension to be made softer or stiffer depending on the terrain. This is reflective of its real-life counterparts, as real trophy trucks have to use the same suspension setup, albeit larger and far more advanced, per the regulations (1)  This system is used for multiple reasons – for starters, independent suspension can offer greater travel on the y-axis, which allows for both a smoother ride, but also means that the vehicle can drive on surfaces with greater changes in elevation, such as rock crawling or sand-dunes. Because each wheel has its own suspension linkage, it also means that the vehicle can traverse larger obstacles without its centre of gravity changing too much. Another benefit is that if the linkage of one wheel is damaged, it can be replaced on its own instead of having to change the linkage for the whole end of the vehicle. Using shock absorbers prevents the vehicle from being too stiff, which makes the car more capable and less prone to damage via vibration or shock impact. Because the shock absorbers on our unit are adjustable, we can change it dependent on the conditions. Stiffer suspension is more suitable for flatter terrain, as it means the vehicle lays flatter when driving around corners at a high speed, and makes it less prone to tipping over, which is one of the main flaws of softer suspension setups. On the other hand, on tougher terrain, softer suspension is more desirable because it allows the vehicle to travel over harsher obstacles without tipping over.

References:

- SCORE International Off-Road Racing Rules and Regulations 2011-2015.